# Hazard Identification Systems

Key Concepts

- I can identify the three types of Hazardous Information Systems.

- I can identify the danger posed by a chemical or product based on the warning label.

Hazard Identification System (HIS)

- colour-coded set of four boxes arranged either as a diamond or box

- blue = health, red = flammability, yellow = reactivity, white = specific hazards

- uses a scale of 0-4 to indicate the hazard level

- 0 = no hazard

- 1 = low hazard risk

- 2 = medium hazard risk

- 3 = high hazard risk

- 4 = extreme hazard risk

Fire Hazard (Red)

# F

Health Hazard (Blue)

R

H

Reactivity Hazard (Yellow)

SC

Specific Hazard (White)

### Health (Blue)

4	Can cause death or major injury despite medical treatment.

3	Can cause serious injury despite medical treatment.

2	Can cause injury; requires prompt treatment.

1	Can cause irritation if not treated.

0	No hazard.

#### Fire (Red)

4	Very flammable gases or very volatile flammable liquids.

3	Can be ignited at all normal temperatures.

2	Ignites if moderately heated.

1	Ignites after considerable preheating.

0	Will not burn

#### Reactivity / Stability (Yellow)

4	Readily detonates or explodes.

3	Can detonate or explode, but requires a strong initiating force or heating under confinement.

2	Normally unstable but will not detonate.

1	Normally stable; unstable at high temperature and pressure; reacts with water.

0	Normally stable; not reactive with water.

## Special Comments (White)

C	May be carcinogenic with chronic exposure (cancer causing)

SC	Suspected carcinogen

Exp	Risk of explosion

W	Reacts with water

Cor	corrosive

Oxy	oxidizing agent

T	toxic

Household Hazardous Products Symbols (HHPS)

- uses a set of four symbols and two borders to identify potential dangers

| poisonous | flammable | explosive | corrosive |
| --- | --- | --- | --- |
| Danger posed by container. |  | Danger posed by contents. |  |

- Poisonous		ingesting or smelling will cause illness or death

- Flammable		product or fumes easily lights on fire

- Explosive		product or container can explode if heated or punctured

- Corrosive		contents cause chemical burns

WHMIS 2015 (GHS)

- Workplace Hazardous Materials Information System updated in 2015

- provides detailed information about how to store, handle, disposal and first aid of chemicals

- information listed on a Material Safety Data Sheet (MSDS)

Biohazardous Infectious Material

Environmental Toxin

Immediate Toxic to Health

Flammable Oxidizer

Flammable

Corrosive

Less Serious Health Issues or Ozone Depletion

Serious Toxic to Health With Prolonged Exposure

Compressed Gas

Explosive or Extreme Reactivity

Assignment

SDS Activity

HIS Worksheet

Page 90 Q 1, 3

Page 93 Q 1-3