Life Sciences 11

Microscopes

Four Types

- Dissecting Microscope

- 2-4x magnification

- used for a more detailed study of a surface, or to cut, probe or observe a specimen

- Compound Microscope

- up to 2000x magnification with oil immersions lens

- used to observe semi-transparent microorganisms

- may observe bacteria

- Transmission Electron Microscope (1931)

- more than 50 million times magnification

- uses a beam of electrons passed through an ultra-thin specimen

- resolution of 0.2 nm

- Scanning Electron Microscope

- 1-2 million times magnification

- make highly detailed 3-D image of the surface of an organism or cell

- resolution of 0.1 nm

Definitions

- Field of View

- the amount of area seen when looking through the eyepiece of a microscope

- Depth of Field

- the thickness amount of an object that can be kept in focus at one time

- Magnification

- factor by which the apparent size of an object has increased

- Resolution / Resolving Power

- the ability to distinguish between separate objects that are close together

Magnification Calculations

- Magnification

- how many times larger does the image appear due to the power of the magnifying lenses

- calculated by multiplying the power of the ocular lens and the power of the objective lens

Magnification= ocular lens × objective lens

Mag= ocular × objective

- Actual Size

- how big the specimen really is

- calculated by estimating the percentage of the diameter of the field of view he specimen covers and multiply by the actual diameter of the field of view

Actual Size=% field of view covered × diameter of field of view

AS=% fov × fov

- Drawing Magnification

- how many times larger the drawing or picture is compared to the specimen’s actual size

- calculated by dividing the drawing size by the actual size of the specimen as measured in the same units

Drawing Magnification= Drawing SizeActual Size

DM= DSAS